University of Asia Pacific

Department of Computer Science and Engineering

Course Code  : CSE 450 (VLSI Lab)

Mid-Term Lab Examination | Group B | Spring 2026

| Duration | 1 hr | Total Marks | 20 |
| --- | --- | --- | --- |
| Student Name | ______________________ | ID | ________________ |

Mid-Term Task: 4-bit ALU

Objective. Design and implement a 4-bit Arithmetic Logic Unit (ALU) in VHDL that supports exactly three operations: AND, OR,, and ADD. The ALU must be built from separate, reusable modules (logic units, a 4-bit adder, and a multiplexer) and combined in a top-level entity.

System Architecture

A(3:0), B(3:0)

│

┌──────────────┼──────────────┐

▼              ▼              ▼

┌───────────┐  ┌───────────┐  ┌───────────┐

│    AND    │  │     OR    │  │   4-bit   │

│   unit    │  │    unit   │  │   Adder   │──► COUT

└─────┬─────┘  └─────┬─────┘  └─────┬─────┘

│ 00           │ 01           │ 10

▼              ▼              ▼

┌─────────────────────────────────────────┐

│               4-to-1 MUX                │◄── SEL(1:0)

└────────────────────┬────────────────────┘

▼

Y(3:0)

Functional requirement. The 2-bit select input SEL chooses which result is passed to the output Y. The AND and OR units operate bitwise on A and B, while the adder produces a 4-bit sum together with a carry-out. The ALU is purely combinational: Y and COUT must change whenever A, B, or SEL change. COUT must be 0 for every operation other than ADD.

Required Operation

| SEL(1:0) | Operation | Y(3:0) | COUT |
| --- | --- | --- | --- |
| 00 | AND | Y ← A AND B | 0 |
| 01 | OR | Y ← A OR B | 0 |
| 10 | ADD | Y ← (A + B) mod 16 | Carry-out of the 4-bit addition |
| 11 | Unused | Y ← 0000 | 0 |

Required Test Sequence

Your testbench must apply the following sequence. Allow each input combination to settle (for example, wait for 10 ns) before checking the outputs.

| Step | SEL | A | B | Operation | Expected Y | COUT | Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 00 | 1100 | 1010 | A AND B | 1000 | 0 | Bitwise AND |
| 2 | 00 | 1111 | 0101 | A AND B | 0101 | 0 | AND with all-ones passes B |
| 3 | 01 | 1100 | 0011 | A OR B | 1111 | 0 | Bitwise OR |
| 4 | 01 | 1001 | 0100 | A OR B | 1101 | 0 | Bitwise OR |
| 5 | 10 | 0011 | 0101 | 3 + 5 | 1000 | 0 | Simple addition |
| 6 | 10 | 1111 | 0001 | 15 + 1 | 0000* | 1 | 4-bit overflow |
| 7 | 10 | 1010 | 0101 | 10 + 5 | 1111 | 0 | Maximum 4-bit result |
| 8 | 11 | 1010 | 0101 | Unused | 0000 | 0 | Invalid select code |

* Note: For Step 6, the mathematical result is 16, but a 4-bit output can represent only 0–15. Therefore, Y = 0000 and the discarded carry appears on COUT = 1.

Implementation Requirements

- Use VHDL and IEEE.STD_LOGIC_1164.ALL.

- Ports: A, B : in STD_LOGIC_VECTOR(3 downto 0); SEL : in STD_LOGIC_VECTOR(1 downto 0); Y : out STD_LOGIC_VECTOR(3 downto 0); COUT : out STD_LOGIC.

- Build the 4-bit adder structurally from 1-bit full adders (ripple-carry). Do not use the "+" operator or an integer/numeric_std based adder as a replacement.

- The design must be modular: separate entities for the AND unit, OR unit, full adder, 4-bit adder, and 4-to-1 multiplexer, connected by a structural top-level ALU.

- The ALU must be combinational; no clock or process-based registers are allowed.

- SEL = "11" must produce Y = "0000" and COUT = '0'.

- The submitted testbench must apply all eight steps of the required test sequence and use ASSERT statements to compare the actual output with the expected output.

Submission

While submitting to the classroom prepare the report from the third page of this template including the current template in the first and second page. From the third page write your report including but not limited to the following points.

- Attach your entity code and testbench code

- Attach your output visuals (waveform showing all eight test steps)

- Challenges faced during the experiment

- Conclusion